ENGLISH PROGRAMMING

How and when to disable the timing advance during deceleration and when idling

On some vehicles it is convenient to disable the timing advance during deceleration and when idling to avoid jumps or irregular function.

On the other hand, the timing advance is required immediately during acceleration to improve performance, consumption and to minimize the risk of backfiring. With the 510N timing advance processor, the timing advance can be enabled or disabled automatically connecting the BLUE-BLACK wire on the timing advance processor to the throttle potentiometer.

NOTE: do not carry out the timing advance enabling calibration procedure if the BLUE-BLACK wire is not connected.

TIMING ADVANCE ENABLING CALIBRATION PROCEDURE

The throttle potentiometer signal is not always the same; therefore, the trip point needs to be calibrated.

Adjustment is carried out during GAS function by acting on the timing advance trip register as follows:

1) Ensure the register is turned completely clockwise.

2) While the vehicle is idling, turn the register anticlockwise until the RED LED switches off (timing advance disabled).

3) With this adjustment, during acceleration, the RED LED on the timing advance processor lights up and then switches off when the accelerator is released.